4.3 Electric circuits: Cambridge IGCSE Physics worksheet

1.  Two identical lamps are connected in parallel across a battery. One lamp breaks and stops conducting.

What happens to the other lamp?

A	It goes out because the whole circuit is broken.

B	It stays lit with the same brightness.

C	It becomes dimmer because the battery potential difference halves.

D	It becomes brighter because it receives the broken lamp's current.

(1 mark)

2. A second resistor is added in parallel with a resistor connected to a fixed-voltage supply. What happens to the total resistance and the total current from the supply? Total resistance; Total current. A; increases; decreases. B; increases; increases. C; decreases; decreases. D; decreases; increases. (1 mark)

3.  Three identical resistors are connected in series across a 12 V supply.

What is the potential difference across one resistor?

A	2 V

B	3 V

C	4 V

D	12 V

(1 mark)

4.  Three 1.5 V cells are connected in series, all in the same direction.

What is the total potential difference?

A	0.5 V

B	1.5 V

C	3.0 V

D	4.5 V

(1 mark)

5. The battery and meters are ideal. What is the reading on the voltmeter? A 0 V B 3 V C 6 V D 9 V (1 mark)

6. Which row gives one correct rule for a series circuit and one correct rule for a parallel circuit? Series circuit; Parallel circuit. A; current divides between components; current is the same everywhere. B; potential difference is the same across every component; branch potential differences add to the supply. C; current is the same at every point; potential difference is the same across each branch. D; total resistance is less than the smallest resistance; resistances add to give the total. (1 mark)

7.  Currents of 0.25 A and 0.35 A leave a junction along two branches.

What current enters the junction?

A	0.60 A

B	0.10 A

C	0.30 A

D	1.40 A

(1 mark)

8.  A 4 Ω resistor is connected in series with two 4 Ω resistors connected in parallel.

What is the total resistance of the network?

A	2 Ω

B	6 Ω

C	8 Ω

D	12 Ω

(1 mark)

9. The ammeter reads 1.0 A. What are the readings on V1 and V2? V1 reading / V; V2 reading / V. A; 3.0; 6.0. B; 6.0; 3.0. C; 9.0; 9.0. D; 2.0; 1.0. (1 mark)

10.  A 3 Ω resistor and a 7 Ω resistor are connected in series.

What is their combined resistance?

A	10 Ω

B	4 Ω

C	21 Ω

D	2.1 Ω

(1 mark)

11. Use the branch resistance values shown. How does the reading on A1 compare with the reading on A2? A A1 is half A2. B A1 is equal to A2. C A1 is twice A2. D A1 is four times A2. (1 mark)

12. The thermistor is warmed while the supply remains unchanged. What happens to the reading on the main ammeter? A It decreases. B It increases. C It remains constant. D It becomes zero. (1 mark)

13.  Use the circuit shown. State what the ammeter measures and how its reading is related to the two branch currents.

.....................................................................................................

.....................................................................................................

.....................................................................................................

.....................................................................................................

(2 marks)

14. The two labelled resistors form one loop in the circuit shown. Calculate their combined resistance. ..................................................................................................... ..................................................................................................... ..................................................................................................... ..................................................................................................... (2 marks)

15.  State how the current through components connected in series compares at different points in the circuit.

.....................................................................................................

.....................................................................................................

(1 mark)

16. The circuit shown contains a single conducting loop. Name the arrangement of the lamps and compare their currents. ..................................................................................................... ..................................................................................................... ..................................................................................................... ..................................................................................................... (2 marks)

17. The ammeter shows the current in one branch of a parallel circuit. A second branch carries 0.25 A. Calculate the current supplied to the two branches. ..................................................................................................... ..................................................................................................... ..................................................................................................... ..................................................................................................... (2 marks)

18.  A circuit has separate branches connected between two junctions. Name this circuit arrangement.

.....................................................................................................

.....................................................................................................

(1 mark)

19.  In a series circuit, does charge have one path or more than one path?

.....................................................................................................

.....................................................................................................

(1 mark)

20.  State which ammeter measures the total current and which ammeter measures the current in the upper branch.

.....................................................................................................

.....................................................................................................

.....................................................................................................

.....................................................................................................

(2 marks)

21. The two ammeters are placed at different points in the same series circuit. Use the readings shown to identify why they cannot both be correct. ..................................................................................................... ..................................................................................................... ..................................................................................................... ..................................................................................................... (2 marks)

22. The diagram below shows a potential divider made from two resistors in series: R1 = 3 kΩ and R2 = 9 kΩ. The voltmeter measures the output potential difference across R2.

(a) State what a potential divider does.

.....................................................................................................

.....................................................................................................

(1 mark)

(b) Calculate the total resistance of the circuit.

.....................................................................................................

.....................................................................................................

(1 mark)

(c) Calculate the current in the circuit. Give your answer in mA.

.....................................................................................................

.....................................................................................................

.....................................................................................................

.....................................................................................................

(2 marks)

(d) Calculate the output potential difference across R2.

.....................................................................................................

.....................................................................................................

.....................................................................................................

.....................................................................................................

(2 marks)

(e) Explain why the output p.d. is greater than the p.d. across R1.

.....................................................................................................

.....................................................................................................

.....................................................................................................

.....................................................................................................

(2 marks)

23. The circuit has two parallel branches. Branch 1 contains R1 = 6 Ω only. Branch 2 contains R2 = 4 Ω and R3 = 8 Ω in series.

(a) State the potential difference across each branch.

.....................................................................................................

.....................................................................................................

(1 mark)

(b) Calculate the total resistance of branch 2.

.....................................................................................................

.....................................................................................................

.....................................................................................................

.....................................................................................................

(2 marks)

(c) Calculate the current through R1.

.....................................................................................................

.....................................................................................................

.....................................................................................................

.....................................................................................................

(2 marks)

(d) Calculate the current through branch 2.

.....................................................................................................

.....................................................................................................

.....................................................................................................

.....................................................................................................

(2 marks)

(e) Calculate the total current from the battery.

.....................................................................................................

.....................................................................................................

.....................................................................................................

.....................................................................................................

(2 marks)

(f) Calculate the total resistance of the whole circuit.

.....................................................................................................

.....................................................................................................

.....................................................................................................

.....................................................................................................

(2 marks)

(g) Calculate the potential difference across R2 and the potential difference across R3.

.....................................................................................................

.....................................................................................................

.....................................................................................................

.....................................................................................................

.....................................................................................................

(4 marks)