4.4 Electrical safety: Cambridge IGCSE Physics worksheet - Mark Scheme

| Question | Answer | Marks |
| --- | --- | --- |
| 1 | A circuit breaker can be reset, whereas a melted fuse must be replaced. | (1 mark) |
| 2 | The earth wire provides a low-resistance path for the fault current to flow to earth, preventing the case from remaining live. | (2 marks) |
| 3 | The fuse wire heats until it melts, breaking the circuit. | (2 marks) |
| 4 | Any one from: the user cannot get an electric shock from the case because it is non-conducting and no accessible metal part can become live; it does not need an earth wire. | (1 mark) |
| 5 | A person could touch an exposed live wire and receive an electric shock. | (1 mark) |
| 6 | Moisture lowers the resistance of the skin, so a larger current can flow through the body. | (2 marks) |
| 7 | The cable can overheat and the hot cable can cause a fire. | (2 marks) |
| 8 | A 3 A fuse. | (1 mark) |
| 9 | Disconnect the lamp from the mains and do not use it because the exposed connection could be live and cause an electric shock. | (2 marks) |
| 10 | The extension lead may overheat because the combined current is too large. | (2 marks) |
| 11 | It disconnects the supply, stopping current in the faulty circuit. | (1 mark) |
| 12 | The current could become large enough to overheat the cable without melting the fuse. | (2 marks) |
| 13 | The live wire, so a melted fuse disconnects the appliance from the live supply. | (2 marks) |
| 14(a) | Use I=VR. I=2300.40. I=575 A. | (3 marks) |
| 14(b) | 575 A is much greater than the 13 A fuse rating. | (1 mark) |
| 14(c) | The live wire makes the metal case live, giving the case a potential difference relative to earth. The earth wire provides a low-resistance path to earth. A very large current flows in the earth wire and live wire. The fuse wire melts or blows, breaking the circuit and disconnecting the appliance from the mains supply.​ | (4 marks) |
| 15(a) | The total power is 2300+920+1380=4600 W. Use I=PV. I=4600230=20 A. | (3 marks) |
| 15(b) | The current is greater than the 13 A rating of the extension lead, so it is overloaded. Excess current can cause overheating of cables, melting insulation or fire. | (2 marks) |
| 15(c) | Damaged insulation. Damp conditions. Accept overheating cables or exposed conductors. | (2 marks) |
| 15(d) | amaged insulation or exposed conductors: a person could touch the exposed live wire. Damp conditions: water conducts, so it can make a conducting path between the live wire and a person. Current then flows from the live wire through the person to earth, causing an electric shock.​ | (3 marks) |
| 16(a) | Use I=PV. I=800230=3.48 A, so the current is about 3.5 A. | (2 marks) |
| 16(b) | Choose a 5 A fuse for each drill. 5 A is the lowest available rating above the normal current of about 3.5 A. A 3 A fuse is too low, and a 13 A fuse is unnecessarily high. | (2 marks) |
| 16(c) | If the live wire touches the metal case, the case could become live. The earth wire provides a low-resistance path to earth. The large fault current causes the fuse to melt or a trip switch to operate, disconnecting the supply and reducing the risk of electric shock. | (3 marks) |
| 16(d) | ouble insulated means there are two layers of insulation, such as insulation around wires and a non-conducting plastic outer case. There is no exposed conducting metal case that can become live, so an earth wire is not needed. | (2 marks) |
| 16(e) | A fuse without an earth wire can still protect the circuit and cabling from excess current, a short circuit or overloading. If the current becomes too large, the fuse melts and breaks the circuit, helping to prevent overheating, melted insulation or fire. | (2 marks) |